| 360 Accessibility Audit Section 2 – Getting in your building |  |  |
| --- | --- | --- |
| Completed by |  |  |
| The first problem many disabled people encounter is finding and getting into a building. Clear and useful access information on your website is important with information about public transport, accessible parking and routes to the building entrance. (See the ‘Creating an accessibility area on your website guidance that is part of this pack). Your goal is to create routes and entrances that enable disabled people to access your building as independently as possible - step free routes with solid surfaces, easy to open doors, space for wheelchairs to navigate easily, suitable handrails, good lighting and colour contrast etc. The best way to discover the shortcomings of your building is to ask a number of disabled people to show you the problems and barriers they encounter. You will find that many disabled people in your local community will be happy to help you learn about the accessibility challenges your building offers. You will learn a lot from your congregational survey. Take time to investigate the issues that are raised and consult on possible solutions. |  |  |
| Question |  | Comment and action |
| How close can people park to your building? |  |  |
| Are there any priority street parking bays nearby? |  |  |
| If you have a church carpark, are bays reserved for disabled drivers and/or blue badge parking? |  |  |
| If you have priority parking bays, are they clearly marked? Are the markings wide enough for wheelchair alongside the vehicle? |  |  |
| Note: Disabled parking spaces should be a minimum of 2.4m wide x 4.8m long and have a side hatched area, 1.2m wide to provide additional room for people with mobility problems or wheelchair users to be able to access their vehicle. Spaces should be clearly marked and signed. Where bays are proposed adjacent to each other, a 2.4m width can be provided together with a shared space of 1.2m demarked between the spaces. See: https://assets.publishing.service.gov.uk/media/67b71a720495029f3e1c6a81/Car_parking_Redacted.pdf |  |  |
| Is there a level/step free route from any disabled/priority parking bays to your entrance? |  |  |
| How far is the route and is it navigable by wheelchair or by those using sticks and walking aids? |  |  |

| Are there drop kerbs in convenient and suitable places near the building to make access for wheelchair users and others as easy as possible? |  |
| --- | --- |
| What are the paths within your curtilage like? Are they gravel or solid surfaces? Do they drain well when it is wet or are there puddles? Do they get slippery? Do leaves accumulate on paths in the autumn? |  |
| Is there a step from the path/road onto the church grounds? |  |
| Is there a gate to open from the road into the churchyard? Can it be opened by wheelchair users and those with restricted strength or movement? |  |
| Is the gate wide enough for a wheelchair user to negotiate unaided? |  |
| Is entrance to the building step free? If not, do you have portable ramps to facilitate this? |  |
| Note: If you claim it is ‘step free,’ make sure there are no ridges/high thresholds or small steps/changes of level that you have overlooked. A threshold that an ambulant person doesn’t notice can be a barrier to a wheelchair user and a trip hazard to others. Thresholds should be no higher than 15mm. Inset doormats can present difficulty because of changes of level and/or texture. |  |
| If entrance is step free, is this through the main entrance or an alternative entrance? |  |
| If it is an alternative entrance, is this well signposted and is it always usable or does it need someone to facilitate access? |  |
| If access has to be facilitated, how is this done and is this made clear via signage. Can a phone number or intercom be provided to call for assistance? |  |
| Do all steps and slopes/ramps have properly positioned and contrasting handrails on both sides? |  |
| Note: Some people who can walk but have restricted mobility find slopes/ramps easier than steps, others prefer steps with sturdy handrails. Handrails on both sides enable people to use whichever hand offers them most support. Good colour contrast for handrails helps visually impaired people find the way. |  |

| Are steps suitably wide and even and are risers easy to negotiate? Are treads level and solid? |  |
| --- | --- |
| Do top steps have contrasting edges and other steps contrasting strips (nosings)? |  |
| If there is a wide area that leads to a step, is there any tactile strip before the step? |  |
| Are ramps a maximum 1 in 12 gradient, or preferred between 1 in 16 and 1 in 20? Do they have nonslip surfaces? |  |
| Is there outside lighting that lights the steps and ramps sufficiently? |  |
| Is the entrance obvious and clearly signposted? When approaching the building is it clear which door to approach? |  |
| Is there any contrast to make the doorway clear for those with visual impairment? |  |
| Do you have glass doors? If so, are there sightline strips (reveals) to stop people walking into them? |  |
| If the doors open outwards, is there sufficient space at the top of any steps for wheelchair users to be safe as the door is opening? |  |
| If solid doors, do they have sight panels. Are there panels low enough for wheelchairs to be seen? |  |
| Note: It is often not possible to install sight panels in old solid Church doors. However, all internal doors should have sight panels installed at an upper and lower level to enable the observation of taller and smaller and standing and seated people who might be behind the doors. |  |
| Is there a single door or double doors? |  |
| If double, do both doors need to be opened for wheelchair access, access with a walking frame or for someone to be helped/guided through the doorway? |  |

| If double, do you keep both doors open or unbolted? |  |
| --- | --- |
| Do the doors have closers? If so, do these increase the difficulty some might have getting in? If the doors are heavy, can this be made easier? |  |
| Can entrance doors be opened without assistance by a wheelchair user or someone with restricted strength and movement? |  |
| Is there still easy access once the service has started and the welcome team are no longer on duty? |  |
| Could a call bell or intercom or phone number be provided for anyone who might need to call for assistance? |  |